Network Layers

- Application Layer (Layer 7)

  - The layer that users / applications interact with, it generates HTTP requests, Email requests (SMTP) or file transfers etc..

  - Its job is to create the request into a valid format

  - Security: This layer is where WAF sits it scans the data in this layer before it is packed and sent across the network

  - Examples: HTTP, HTTPS, SMTP (Email), SSH

- Presentation Layer (Layer 6)

  - This layer formats data so that receiver could understand it

    - Example:

  - Security: This layer also handles encryption of Data if HTTPS is being used

  - Examples: SSL/TLS (Encryption), JPEG, ASCII, JSON formatting

- Session Layer (Layer 5)

  - The "Manager." It establishes, maintains, and terminates connections between applications. It keeps different data streams separate (so your YouTube tab doesn't mix with your Email tab)

  - Example: APIs, Sockets

- Transport Layer (Layer 4)

  - The "Postman." It ensures data arrives reliably and completely. It breaks large data into small "segments." It uses Ports to direct traffic to the right service (Web vs. Email)

  - Security: in terms of security Standard network Firewalls sit here to block port 22 or 88 for example

- Network Layer (Layer 3)

  - The "Navigator." It handles Routing. It adds IP Addresses to packets to figure out the best path across the internet to reach the destination

  - Security: Routers and Network Firewalls work here (blocking specific IP addresses)

  - Examples: IP Addresses (IPv4, IPv6), Routers, ICMP (Ping)

- Data Link Layer (Layer 2)

  - The "Local Delivery." It moves data between devices on the same network (like your laptop to your Wi-Fi router). It uses MAC Addresses (hardware IDs)

  - Example: MAC Addresses

- Physical Layer (Layer 1)

  - The "Raw Physics." It converts binary data (1s and 0s) into electrical signals, light pulses, or radio waves

  - Example: Ethernet cables, Fiber Optics

Why is WAF at layer 7 example

Attacker tries to use SQL injection